Computation Set 03 Solution

AAE 440: Spacecraft Attitude Dynamics

Spring 2022

Due Date: April 06, 2022

Brendan Gillis

Problem 01: Problem Statement

Problem 01: Problem Solution

Part (a):

Part (a.1):

To find our CRP ρ at our initial orientation we can use our Euler Angles to compute the [BN] DCM.  Then we can use that to compute the CRP.

For a given i-j-k sequence of Euler Angles θ1, θ2, θ3. The DCM can be written as:

C=Rkθ3Rjθ2Ri(θ1)

ρ=1TraceC+1C23-C32C31-C13C12-C21

ρ=0.4190.066-0.492

Part (a.2):

To compute the time derivative of the CRP we can apply the relationship shown in the lectures.  Here the angular velocity is written in the B frame and the CRP and CRP derivative are written in both the B and N frames.  This is because ρ is defined using the principal rotation parameters, whose single axis of rotation causes the PRP to be the same in both frames.

ρ1ρ2ρ3=121+ρ12ρ1ρ2-ρ3ρ1ρ3+ρ2ρ2ρ1+ρ31+ρ22ρ2ρ3-ρ1ρ3ρ1-ρ2ρ3ρ2+ρ11+ρ32ω1ω2ω3

Part (a.3):

Here I defined a function that incorporates the variable angular velocity as it calls our KDE_CRP() function. Using this we can call our integrator to integrate the CRP history over time.

Part (a.4):

Here we can take our CRP history and convert this to PRP by nesting our previously written functions DCMtoPRP( CRPtoDCM( CRP ) ).  Here we can see that our PRP angle θ approaches π.  Additionally, we know our CRP has a singularity at θ=π+2πn (caused by the tanθ2 in our CRP calculation).  This is why the results shown in Part (a.3) start to blow up towards the end of the simulation.

C=11+ρ⋅ρ1-ρ⋅ρI3x3+2ρρ-2ρ

cosθ=12C11+C22+C33-1,         λ=λ1λ2λ3=12sin⁡(θ)C23-C32C31-C13C12-C21

Part (a.5):

Here we can convert our previously calculate CRP into EP by first converting them to a DCM, then converting the DCM to EP.  Here we can see that the two results are identical

ϵ12=141+2C11-TraceC,  ϵ32=141+2C33-TraceC

ϵ22=141+2C22-TraceC,  ϵ42=141+TraceC

ϵ1ϵ2=C12+C214,  ϵ1ϵ4=C23-C324

ϵ2ϵ3=C23+C324,  ϵ2ϵ4=C31-C134

ϵ3ϵ1=C31+C134,  ϵ3ϵ4=C12-C214

Part (a.6):

I was unable to numerically integrate this system over the time span t=0 to t=100 using our CRP kinematic differential equations.  This is because CRP are calculated as:

ρ=λtan⁡(θ2)

This causes the CRP to have a singularity at θ=π+2πn.  As is shown in Part (a.3), the satellite’s angle approaches π at 10 seconds causes the numerical integration to hit a singularity.  The run up to this can be seen by the significant increase in ‖ρ‖ as θ→π.

Part (b):

Part (b.1):

After calculating our DCM in Part (a.1), we can convert the DCM to our MRP by applying the following relationship:

σ=1d(d+2)C23-C32C31-C13C12-C21,  d=traceC+1

σ=0.1910.03-0.224

Part (b.2):

Here we can calculate time derivative of the MRP by applying the equation shown in the lectures. The angular velocity is written in the B frame and the MRP and MRP derivative are written in both the B and N frames.  This is because σ is defined using the principal rotation parameters, whose single axis of rotation causes the PRP to be the same in both frames.

σ1σ2σ3=141-σ2+2σ122(σ1σ2-σ3)2(σ1σ3+σ2)2(σ2σ1+σ3)1-σ2+2σ222(σ2σ3-σ1)2(σ3σ1-σ2)2(σ3σ2+σ1)1-σ2+2σ32ω1ω2ω3

Part (b.3):

Here I defined a function that incorporates the variable angular velocity as it calls our KDE_MRP() function. Using this we can call our integrator to integrate the MRP history over time.

Part (b.4):

To convert our calculated MRP values to PRP, we first convert them to a DCM then convert the DCM to PRP.

C=I3+8σσ-41-σ⋅σσ1+σ⋅σ2

cosθ=12C11+C22+C33-1,         λ=λ1λ2λ3=12sin⁡(θ)C23-C32C31-C13C12-C21

Part (b.5):

Here we can convert our previously calculate MRP into EP by first converting them to a DCM, then converting the DCM to EP.  Here we can see that the two results are identical

ϵ12=141+2C11-TraceC,  ϵ32=141+2C33-TraceC

ϵ22=141+2C22-TraceC,  ϵ42=141+TraceC

ϵ1ϵ2=C12+C214,  ϵ1ϵ4=C23-C324

ϵ2ϵ3=C23+C324,  ϵ2ϵ4=C31-C134

ϵ3ϵ1=C31+C134,  ϵ3ϵ4=C12-C214

Part (b.6):

Looking at the time history of the MRP we see that it significantly exceeds unity.  Because σ=λtanθ4, this value greater than unity indicates that the angle is between (π, 2π).  Additionally, by comparing the results of Part (b) to Part (a) we can see a significant advantage of MRP over CRP, mainly that it’s singularity of 2π+4πn offers twice the working range within any integration.  In this problem  the additional headroom between singularities enables is to fully integrate from t=0 to t=100, something we could not do in Part (b) due to the PRP angle increasing past π.

Problem 02: Problem Statement

Problem 02: Problem Solution

Part (a):

The derivative of the angular velocity vector can be computed using the equation below.  Here the angular velocity, its derivative and the inertia tensor are all expressed in the B frame.

ω1=-I3-I2I1ω2ω3ω2=-I1-I3I2ω3ω1ω3=-I2-I1I3ω1ω2

Part (b):

Here I implemented a function satellite_orientation_omeag() that returns dwdt and incorporates the moments of inertia.  We can integrate this function from t=0 to t=100 using the given initial angular velocities.

Part (c):

Here

Part (d):

Here

Part (e):

Here

Part (f):

Here