Computation Set 04 Solution

AAE 440: Spacecraft Attitude Dynamics

Spring 2022

Due Date: April 13, 2022

Brendan Gillis

Problem 01: Problem Statement

Problem 01: Problem Solution

Part (a):

Using our function dwdt_torqueFree() we can numerically integrate this system given initial angular velocity and the inertia tensor in the B frame.

Part (b):

Using the same code as in Part (a) we can compute the angular velocity of the axisymmetric body and compare it to the non-axisymmetric geometry.

Part (c):

Comparing the two results we can see that the non-axisymmetric body has a variable and periodic ω3 compared to the axisymmetric body’s constant ω3.  Additionally, the periodic frequency of oscillation for ω1 and ω2 is higher in the non-axisymmetric case. Lastly, we can compare the peak values for the angular velocity noticing that for the non-axisymmetric body ω2 and ω3 have larger maximums.

|  | ω1, max | ω2, max | ω3, max |
| --- | --- | --- | --- |
| Axisymmetric Body | 0.112 | 0.112 | 0.1 |
| Non-Axisymmetric Body | 0.11 | 0.117 | 0.103 |

Part (d):

We can compute the magnitude of the angular momentum, and the rotation energy of the satellite by using the equations below.  Using the graphs, we can confirm that both quantities are constant over time, this is something we expect for a body in a torque free environment.

H2= BI ω2,  Trot=12ω BI ω

Part (e):

Using our ω history we can compute the ω history with our dwdt_torqueFree() function. This can then be used to calculate our constants. Looking at the graph we can see that Ci is constant (axis scaled by 1e-12) with the only variation being numerical error.

Part (f):

Part (f.1):

To compute the MRP history we can construct a state variable and compute its derivative:

x=ωσ,  x=ωσ=dwdt_torqueFree(ω, I)KDE_MRP(σ, ω)

Part (f.2):

With our MRP history, we can computer the DCM NB=[BN]T and finally compute our angular momentum in the N frame. Notice that the values are constant and only vary O(1e-10) due to numerical integration error.

NH=[BN]T BH

Part (f.3):

Finally, we can plot our angular velocity in the N frame with our N frame angular momentum and visually confirm that the angular momentum vector is perpendicular to the herpolhode which projects the locus of ω(t).

Part (g):

When comparing the herpolhode of the axisymmetric body to that of the non-symmetric body we can see 2 main differences.  Firstly the two plains do not coincide.  This makes sense because the changes between the Inertia tensors implies they do not have the same angular momentum vector, and thus not the same perpendicular plane.  Additionally, the symmetric body has a simple closed circular loop while the asymmetric body has a more complex and open loop.

Problem 02: Problem Statement

Problem 02: Problem Solution

Part (a):

To confirm the solutions given by Eq. (6) are valid we can plug them into Eq. (7).

Solution 1: ω2t=ω3t=0,  ω1t=ω1o(≠0)

ω1=-I3-I2I1ω2ω3ω2=-I1-I3I2ω3ω1ω3=-I2-I1I3ω1ω2=-I3-I2I10=0=-I1-I3I20ω1=0=-I2-I1I3ω10=0

Solution 2: ω3t=ω1t=0,  ω2t=ω2o(≠0)

ω1=-I3-I2I1ω2ω3ω2=-I1-I3I2ω3ω1ω3=-I2-I1I3ω1ω2=-I3-I2I10ω2=0=-I1-I3I20=0=-I2-I1I3ω20=0

Solution 3: ω1t=ω2t=0,  ω3t=ω3o(≠0)

ω1=-I3-I2I1ω2ω3ω2=-I1-I3I2ω3ω1ω3=-I2-I1I3ω1ω2=-I3-I2I10ω3=0=-I1-I3I20ω3=0=-I2-I1I30=0

Part (b):

Here we know that the maximum energy solution corresponds to Solution 1, where all angular momentum is about the b1 axis which has the lowest moment of inertia. On the flipside, the minimum energy solution is Solution 3, where all angular velocity and momentum is about the b3 axis, the axis with the highest moment of inertia.  Lastly our intermediate energy solution is Solution 2 where all angular momentum is about the b2 axis which has a moment of inertia somewhere between the other two axis. This relationship between moment of inertia and energy makes sense because for a given angular momentum because while H2= BI ω2 is constant, Trot=12ω BI ω will increase as the MOI decreases and ω increases.

Part (c):

We

Part (d):

Using

Part (d.1):

From

Part (d.2):

From

Part (d.3):

From